Division 9 – Finishes
09511 Acoustical Panel Ceilings
Ceiling Tile

Manufacturer: Armstrong

Item 770
09920 Interior Painting
Water Based Catalyzed Epoxy

Manufacturer: The Sherwin-Williams Company

Use in bathrooms – especially residence halls

Finish: Semi-gloss

Sherwin Williams is the preferred paint provider.  Any substitutes will require prior approval through the Facilities Management Planning and Design department in conjunction with the Facilities Management paint shop foreman.

Hallways, stairwells and other public spaces shall have satin finish.

Office areas, reception spaces, or other private department areas shall have egg shell finish.

Door Frames – Sherwin Williams direct to metal (DTM) paint.

Ceilings shall have a flat finish.  White has been the standard but can be decided on a project-by- project basis.

New dry wall shall receive on coat of Sherwin Williams PVA primer and two finish coats.  During a remodel existing dry wall shall receive two finish coats.

Wood Work
Existing wood work should use Diamond Vogel varnish.  Mix one part satin to two parts gloss to create a semi-gloss finish.

New wood work should use a water born product.

Exterior Painting
Concrete should have an elostometric paint either from Sophir Morris or Diamond Vogel.

Metal should receive a polyurethane finish.  Product is Coronado from Sophir Morris.

Parking Lots and ADA Spaces

